# Packet Tracer - Investigate STP Loop Prevention

# Objectives

In this lab, you will observe spanning-tree port states and watch the spanning-tree convergence process.

Describe the operation of Spanning Tree Protocol.

Explain how Spanning Tree Protocol prevents switching loops while allowing redundancy in switched networks.

# Background / Scenario

In this activity you will use Packet Tracer to observe the operation of Spanning Tree Protocol in a simple switched network that has redundant paths.

# Instructions

## Observe a Converged Spanning-Tree Instance

### Verify Connectivity.

Ping from PC1 to PC2 to verify connectivity between the hosts. Your ping should be successful.

### View spanning-tree status on each switch.

Use the show spanning-tree vlan 1 command to gather information about the spanning tree status of each switch. Complete the table. For the purposes of the activity, only consider information about the Gigabit trunk ports. The Fast Ethernet ports are access ports that have end devices connected and are not part of the inter-switch trunk-based spanning tree.

Open configuration window

| Switch | Port | Status (FWD, BLK…) | Root Bridge? |
| --- | --- | --- | --- |
| S1 | G0/1 | Forwarding State | No |
| S1 | G0/2 | Forwarding State | No |
| S2 | G0/1 | Forwarding State | Yes |
| S2 | G0/2 | Forwarding State | Yes |
| S3 | G0/1 | Forwarding State | No |
| S3 | G0/2 | Blocking State | No |

Packet Tracer uses a different link light on one of the connections between the switches.

#### Questions:

What do you think this link light means?

The green light shows that the port is in a forwarding state whereas the orange link light reveals that the port is blocked.

Type your answers here.

What path will frames take from PC1 to PC2?

From PC1 it will use Fa0/1 port of Switch 1 (S1) to send frames. S1 will release them via its G0/2 port then they will get to Switch 2 (S2)’s G0/1 port. S2 will then send the frames to get to P2 using the Fa0/1 port.

Type your answers here.

Why do the frames not travel through S3?

Because the link between S1 and S3 is logically blocked. S3’s G0/2 port is blocked from sending/receiving frames.

Type your answers here.

Why has spanning tree placed a port in blocking state?

To create a loop-free topology.

Close configuration window

## Observe spanning-tree convergence

### Remove the connection between S1 and S2.

Open a CLI window on switch S3 and issue the command show spanning-tree vlan 1. Leave the CLI window open.

Open configuration window

Select the delete tool from the menu bar and click the cable that connects S1 and S2.

### Observe spanning-tree convergence.

Quickly return to the CLI prompt on switch S3 and issue the show spanning-tree vlan 1 command.

Use the up-arrow key to recall the show spanning-tree vlan 1 command and issue it repeatedly until the orange link light on the cable turns green (this could take a bit of time). Observe the status of port G0/2.

#### Question:

What do you see happen to the status of the G0/2 port during this process?

The port switched from a blocking state to a forwarding state but before reaching the forwarding state it needs to start by listening, learning first.

You have observed the transition in port status that occurs as a spanning-tree port moves from blocking to forwarding state.

Verify Connectivity by pinging from PC1 to PC2. Your ping should be successful.

Are any ports showing an orange link light that indicates that the port is in a spanning-tree state other than forwarding? Why or why not?

No, because there are no redundant paths. This proves the spanning-tree is primarily used in a hierarchical design where paths are redundant, to create a loop-free topology.

Type your answers here.

Close configuration window

End of document